
	[image: image1.jpg]




广州粤能科技有限公司
GUANGZHOU YUENENG TECHNOLOGY CO., LTD.

	CUSTOMER:
	

	MODEL NO.:
	YN-DCW12095-16-2003P-6.5

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED
CHECKED
Add: ,132 Shishu Road, Yuexiu District, Guangzhou City, Guangdong China.

Mobile:+86-18924165988            skype:18924165988

WhatsApp: +86-18924165988         Email: cngzyn@163.com 




	Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060hPa) unless the standard condition (Temperature: 25±3℃, Humidity: 60±10%R.H. Atmospheric pressure: 860 ～1060hPa) is regulated to measure.

	1
	Rated Voltage
	3Vo-p; 5Vo-p

	2
	Operating Voltage
	2~4Vo-p; 4~6Vo-p

	3
	Rated Current
	Max.50mA; Max.70mA

	4
	Sound Output at 10cm
	Min. 80dB; Min. 85dB

	5
	Coil Resistance
	16±2Ω; 42±2Ω

	6
	Resonant Frequency
	2048Hz

	7
	Operating Temperature
	-20℃～+60℃

	8
	Store Temperature
	-25℃～+70℃

	9
	Net Weight
	Approx 2.0g

	10
	Tone Nature
	Constant

	11
	Dimension
	Φ12 x H9.5 mm

	12
	R o H S
	Yes

	Dimensions
	Unit: mm 

	

[image: image3.png]



[image: image4.png]




	*Unit: mm; Tolerance: ±0.3mm Except Specified
*Housing Material: Black LCP

*Terminal plate: 3 soldering pads, tin Plating Brass


	Electrical And Acoustical Measuring Condition

	Recommended Driving Circuit
[image: image5.png]5Yp-p

Signal in
>

o

[N4148 A Buzzer

W

470

L 1503199





Resonant frequency, 1/2 duty cycle. Square wave.

Signal amplitude should be large enough to saturate the transistor.

	Recommended Setting


[image: image6.wmf]4

0

c

m

1

0

c

m

M

i

c

.

S

P

L

d

B

测

试

示

意

图

A

m

p

l

i

f

i

e

r

放

大

器

S

M

-

1

6

4

1

F

U

N

C

T

I

O

N

G

E

N

E

R

A

T

O

R

信

号

发

生

仪

P

o

w

e

r

电

源

B

u

z

z

e

r




	Frequency Response
      [image: image7.jpg]i\

1KHz

2KHz 4KHz




可波峰焊 ，工作时间≥1S ； 焊接温度≤230℃ ； 焊接时间≤2S

TEST CONDITION.
Standard Test Condition

:
a) Temperature : +5 ~ +35℃  b) Humidity : 45-85% 
c) Pressure : 860-1060mbar
一般测试条件

:  
a) 温度 : +5 ~ +35℃        b) 湿度 : 45-85%   
c) 气压 : 860-1060mbar

Judgment Test Condition

:
a) Temperature : +25 ± 2℃   b) Humidity : 60-70% 
c) Pressure : 860-1060mbar
争议时测试条件

: 
a) 温度 : +25 ± 2℃         b) 湿度 : 60-70%  
c) 气压 : 860-1060mbar



YNKJ





BUZZER SPECIFICATION









_1234567890.bin

