
	[image: image4.png]Bx

L oo, 12000 EREBEER
q & Uz |pooo| | RBER
0888 &8
© | 85dB r

®
RN SE——
EE | L
| i

| BISAEERE 10CM





[image: image1.jpg]



广州粤能科技有限公司

GUANGZHOU YUENENG TECHNOLOGY CO., LTD.


	CUSTOMER:
	

	MODEL NO.:
	YN-DCW1310-2412 140Ω

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED
CHECKED
Add: ,132 Shishu Road, Yuexiu District, Guangzhou City, Guangdong China.

Mobile:+86-18924165988            skype:18924165988

WhatsApp: +86-18924165988         Email: cngzyn@163.com 




	Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060mbar) unless the standard condition (Temperature: 25±2℃, Humidity: 60%～70 %R.H. Atmospheric pressure: 860 ～1060mbar) is regulated to measure.

	1
	Rated Voltage
	12 Vo-p

	2
	Operating Voltage
	8-15Vo-p

	3
	Rated Current
	Max.40mA@ 5Vp-p 

	4
	Coil Resistance
	140±5Ω

	5
	Sound Output at 10cm
	Min.88dB@at 10cm 5Vp-p 2400Hz

	6
	Resonant Frequency
	2.4kHz±0.5 @square wave

	7
	Operating Temperature
	-20℃～+85℃

	8
	Store Temperature
	-30℃～+80℃

	9
	Net Weight
	Approx 2g

	10
	size
	12.8*12.8*H10mm

	11
	Housing Material 
	LCP 

	12
	Environmental 
Protection Regulation
	RoHS

	Dimensions
	Unit: mm

	 [image: image2.jpg]02|A

210.1






	*Unit: mm; Tolerance: ±0.3mm Except Specified
*Housing Material: Black LCP

*Terminal plate: 2 soldering pads, tin Plating Brass


	3.RECOMMEND DIRVING CIRCUIT
 a


	4.Frequency Response
[image: image3.png]dB
98
92
86
80
74
68
62
56
50
44

10K

5K

500 1K

200Hz






APPROVAL SHEET


FOR


SMD BUZZER











Guangzhou Yueneng Technology Co., Ltd.  

