
	[image: image9.png]200Hz 300 400 500 600 000 1k 2k ELS 4k Sk 6k 7k 8k 18k




[image: image1.jpg]



广州粤能科技有限公司

GUANGZHOU YUENENG TECHNOLOGY CO., LTD.


	CUSTOMER:
	

	MODEL NO.:
	YN-DCW9032-16-2703SB

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED
CHECKED
Add: ,132 Shishu Road, Yuexiu District, Guangzhou City, Guangdong China.

Mobile:+86-18924165988            skype:18924165988

WhatsApp: +86-18924165988         Email: cngzyn@163.com 



	Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060mbar) unless the standard condition (Temperature: 25±2℃, Humidity: 60%～70 %R.H. Atmospheric pressure: 860 ～1060mbar) is regulated to measure.

	1
	Rated Voltage
	3 Vo-p

	2
	Operating Voltage
	2～4Vo-p

	3
	Rated Current
	Max.100mA 

	4
	Sound Output at 10cm
	Min. 85dB

	5
	Coil Resistance
	16±3Ω

	6
	Resonant Frequency
	2731Hz

	7
	Operating Temperature
	-20℃～+70℃

	8
	Store Temperature
	-30℃～+80℃

	9
	Net Weight
	Approx 0.5g

	10
	RoHS
	Yes

	Dimensions
	Unit: mm

	[image: image2.png]o O B e

10.5£0.5







	*Unit: mm; Tolerance: ±0.3mm Except Specified
*Housing Material: Black LCP

*Terminal plate: 2 soldering pads, tin Plating Brass


	Electrical And Acoustical Measuring Condition

	Recommended Driving Circuit
[image: image3.png]IN4148 % Buzzer

Signal in
-
SHpp — AW aseanss

470





Resonant frequency, 1/2 duty cycle. Square wave.

Signal amplitude should be large enough tosaturate

the transistor.
	Recommended Setting

[image: image4.png]





	Frequency Response
3Vo-p 50% duty Square wave,10cm





	Surface mounting condition

5.1 Reflow soldering

Recommendable reflow soldering condition is as follows.
[image: image5.png]Temp.
0

265°C

245°C

180£10°C

4°Csec Max.

4°Cisec Max.

Peak Temp.:
250°C max.

Cooling by air
Pre-heating g

170-190°C

90+ 30sec 10 sec. Max.

Time(Sec)




Note: (1) In automated mounting of the SMD sound transducers on PCB, any bending, expanding

and pulling forces or shocks against the SMD sound transducers shall be kept minimum

to prevent them from electrical failures and mechanical damages of the devices.
(2) In the reflow soldering, too high soldering temperatures and too large temperature

      Gradient such as rapid heating or cooling may cause electrical failures and mechanical

damages of the devices. 

	Reliability Test
After any following tests the part shall meet specifications without any degradation in

appearance and performance except SPL. SPL shall not deviate more than -10 dB

from the initial value

Ordinary Temperature Life Test

The part shall be subjected to 96 hours at 25±10℃. Input rated voltage

Test Resonant frequency, 1/2 duty Square wave.

High Temperature
The part shall be capable of with standing a storage temperature of +85℃ for 96 hours.
Low Temperature Test
    The part shall be capable of with standing a storage temperature of ​-40℃ for 96 hours.
Humidity Test

Temperature:+40℃±3℃   Relative Humidity:90%～95% Duration: 48 hours

and expose to room temperature for 6 hours
Temperature Shock Test
Temperature:60℃ /1hour→ 25℃/3hours→-20℃/1hour→ 25℃/3hours (1cycle)

Total cycle: 10 cycles
Drop Test
Standard Packaging From 75cm(Drop on hard wood or board of 5cm thick, 

three sides, six plain.)
Vibration Test

Vibration:1000cycles /min. Amplitude:1.5mm, Duration: 1 hour in each 3 axes
Reflow Test

Use recommendable reflow soldering condition (as shown in 5.1)
(1) No abnormality should be found after reflow
(2) Good soldering to meet soldering requirements



Note:
As this product is not protected from foreign material entering, please make sure that any foreign materials (e.g. magnetic powder, washing solvent, flux, corrosive gas) do not enter this product in your production processes. The functional degradation (e.g. SPL down) may occur if foreign material enter it.
	Packing

[image: image6.png]2203

oooooooooolooo 0000

@@@@

16:03

38:03





	
[image: image7.wmf]
	
[image: image8.wmf]


APPROVAL SHEET


FOR


SMD BUZZER











Guangzhou Yueneng Technology Co., Ltd.  

_1234567890.bin

_1234567891.bin

