
	[image: image1.jpg]




Guangzhou Yueneng Technology Co., Ltd.

	CUSTOMER:
	

	MODEL NO.:
	YN-DCW9650-27PB-4

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED
CHECKED
Add: ,132 Shishu Road, Yuexiu District, Guangzhou City, Guangdong China.

Mobile:+86-18924165988            skype:18924165988

WhatsApp: +86-18924165988         Email: cngzyn@163.com 




	Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060hPa) unless the standard condition (Temperature: 25±3℃, Humidity: 60±10%R.H. Atmospheric pressure: 860 ～1060hPa) is regulated to measure.

	1
	Rated Voltage
	1.5Vo-p;3Vo-p;5Vo-p

	2
	Operating Voltage
	1-2Vo-p;2-4Vo-p;4~7Vo-p;

	3
	Rated Current
	Max.80mA;Max.80mA;Max.80mA;

	4
	Sound Output at 10cm
	Min. 80dB;Min. 85dB;Min. 85dB

	5
	Coil Resistance
	6.5±2Ω;16±3Ω;30±3Ω;

	6
	Resonant Frequency
	2731Hz

	7
	Operating Temperature
	-20℃～+60℃

	8
	Store Temperature
	-30℃～+70℃

	9
	Net Weight
	Approx 0.8g

	10
	Tone Nature
	Constant

	11
	Dimension
	10.8Φ9.6 x H5 mm

	12
	R o H S
	Yes

	Dimensions
	Unit: mm 

	[image: image2.png]3.5

® )
T Te 3 *®

— v

@A6£05







	*Unit: mm; Tolerance: ±0.3mm Except Specified
*Housing Material: Black LCP

*Terminal plate: 3 soldering pads, tin Plating Brass


	Electrical And Acoustical Measuring Condition

	Recommended Driving Circuit
[image: image3.png]5Yp-p

Signal in
>

o

[N4148 A Buzzer

W

470

L 1503199





Resonant frequency, 1/2 duty cycle. Square wave.

Signal amplitude should be large enough to saturate the transistor.

	Recommended Setting


[image: image4.wmf]4

0

c

m

1

0

c

m

M

i

c

.

S

P

L

d

B

测

试

示

意

图

A

m

p

l

i

f

i

e

r

放

大

器

S

M

-

1

6

4

1

F

U

N

C

T

I

O

N

G

E

N

E

R

A

T

O

R

信

号

发

生

仪

P

o

w

e

r

电

源

B

u

z

z

e

r




	Frequency Response
[image: image5.png]





YNKJ





BUZZER SPECIFICATION











                                               

_1234567890.bin

