
	[image: image4.png](10PCS Foam Tray)

Plastic Bag

A0l 60mam30mn | 1x100PCS=100PCS

Foam Ty

Canton Box | 520mamx420mmz280man | 5x1 000PCS=5,000PCS





[image: image1.jpg]



广州粤能科技有限公司

GUANGZHOU YUENENG TECHNOLOGY CO., LTD.

	CUSTOMER:
	

	MODEL NO.:
	YN-DCY1614-2405P-7.6

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED
CHECKED
Add: ,132 Shishu Road, Yuexiu District, Guangzhou City, Guangdong China.

Mobile:+86-18924165988            WhatsApp: +86-18924165988         

skype:18924165988                 Email: cngzyn@163.com  


	Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060hPa) unless the standard condition (Temperature: 25±3℃, Humidity: 60±10%R.H. Atmospheric pressure: 860 ～1060hPa) is regulated to measure.

	1
	Rated Voltage
	5Vo-p;

	2
	Operating Voltage
	4～6Vo-p;

	3
	Rated Current
	Max.30mA 

	4
	Sound Output at 10cm
	Min. 85dB

	5
	Resonant Frequency
	2.4kHz±0.3

	6
	Operating Temperature
	-20℃～+60℃

	7
	Store Temperature
	-30℃～+70℃

	8
	Net Weight
	Approx 4.7g

	9
	Tone Nature
	Constant

	10
	Dimension
	Φ16*H14 mm

	Dimensions
	Unit: mm 

	[image: image2.png]©16.0£0.5

14.040.5






	*Unit: mm; Tolerance: ±0.3mm Except Specified

*Housing Material: Black LCP

*Terminal plate: 3 soldering pads, tin Plating Brass


3.Frequency Response
[image: image3.png]()

5 8 8 8 g

8 10 12 14 164DC 8 10 12 14 16KVDC>




4.Packing


YNKJ





BUZZER SPECIFICATION












