

	[image: image1.jpg]




GUANGZHOU YUENENG TECHNOLOGY CO., LTD.

	CUSTOMER:
	

	MODEL NO.:
	YN-YDW1707-4012PA10

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED
CHECKED
Add: ,132 Shishu Road, Yuexiu District, Guangzhou City, Guangdong China.

Mobile:+86-18924165988            WhatsApp: +86-18924165988         

skype:18924165988                 Email: cngzyn@163.com  



	Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060hPa) unless the standard condition (Temperature: 25±3℃, Humidity: 60±10%R.H. Atmospheric pressure: 860 ～1060hPa) is regulated to measure.

	1
	Rated Voltage
	3Vo-p；5Vo-p；12Vo-p；    

	2
	Operating Voltage
	1-30Vo-p

	3
	Rated Current
	Max.5mA

	4
	Sound Output at 10cm
	 Min.70dB； Min.75dB；Min.75dB；

	5
	Capacitance at 120Hz
	25±40% n F

	6
	Resonant Frequency
	4000Hz

	7
	Operating Temperature
	-10℃～+60℃

	8
	Store Temperature
	-20℃～+70℃

	9
	Net Weight
	Approx 1.6g

	10
	Tone Nature
	Constant

	11
	Dimension
	Φ17 x H7 mm

	12
	R o H S
	Yes

	Dimensions
	Unit: mm 

	[image: image2.png]10.040.3

417.0£0.3







	*Unit: mm; Tolerance: ±0.3mm Except Specified
*Housing Material: Black LCP

*Terminal plate: 3 soldering pads, tin Plating Brass


	Electrical And Acoustical Measuring Condition

	Recommended Driving Circuit
[image: image3.png]1.32.Standard text Fixture

1.31.Recommended Circuit
the folowing are examples of extemaly diven crcuis. No reaction in space with in 400mmin all drecton

red
buzzer

Square o

signal
generator | |

400mm








	Frequency Response
[image: image4.png]200Hz 300 408 S00 600 6808 1k 2k 3k 4k Sk 6k 7k 8k 18k






	Reliability Test

After any following tests the part shall meet specifications without any degradation in

appearance and performance except SPL. SPL shall not deviate more than -10 dB

from the initial value

Ordinary Temperature Life Test
The part shall be subjected to 96 hours at 25±10℃. Input rated voltage
Resonant frequency, 1/2 duty Square wave.

6.2 High Temperature Test
The part shall be capable of with standing a storage temperature of +85℃ for 96 hours.
6.3 Low Temperature Test 
The part shall be capable of with standing a storage temperature of ​-40℃ for 96 hours.
Humidity Test
Temperature:+40℃±3℃   Relative Humidity:90%～95% Duration: 48 hours

and expose to room temperature for 6 hours
Temperature Shock Test

Temperature:60℃ /1hour→ 25℃/3hours→-20℃/1hour→ 25℃/3hours (1cycle)

Total cycle: 10 cycles
Drop Test

Standard Packaging From 75cm(Drop on hard wood or board of 5cm thick, 

three sides, six plain.)
Vibration Test

Vibration:1000cycles /min. Amplitude:1.5mm, Duration: 1 hour in each 3 axes
Reflow Test

Use recommendable reflow soldering condition (as shown in 5.1)
(1) No abnormality should be found after reflow
(2) Good soldering to meet soldering requirements
Note:
As this product is not protected from foreign material entering, please make sure that any foreign materials (e.g. magnetic powder, washing solvent, flux, corrosive gas) do not enter this product in your production processes. The functional degradation (e.g. SPL down) may occur if foreign material enter it.


YNKJ





BUZZER SPECIFICATION











深圳电子批发商城                                                                       https://shop289834711.taobao.com

