[image: image1.jpg]




	

GUANGZHOU YUENENG TECHNOLOGY CO., LTD.

	CUSTOMER:
	

	MODEL NO.:
	YN-YDW2207-4012P10

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED
CHECKED
Add: ,132 Shishu Road, Yuexiu District, Guangzhou City, Guangdong China.

Mobile:+86-18924165988            skype:18924165988

WhatsApp: +86-18924165988         Email: cngzyn@163.com 




	Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060hPa) unless the standard condition (Temperature: 25±3℃, Humidity: 60±10%R.H. Atmospheric pressure: 860 ～1060hPa) is regulated to measure.

	1
	Rated Voltage
	3Vo-p； 5Vo-p；12Vo-p

	2
	Operating Voltage
	1-30Vo-p

	3
	Rated Current
	Max3mA； Max5mA；Max5mA

	4
	Sound Output at 10cm
	Min.80dB ；Min.85dB ；Min.85dB  

	5
	Resonant Frequency
	4000Hz±0.5

	6
	Operating Temperature
	-10℃～+60℃

	7
	Store Temperature
	-20℃～+70℃

	8
	Net Weight
	Approx 2.5g

	9
	Tone Nature
	Constant

	10
	Dimension
	Φ22 x H7 mm

	11
	R o H S
	Yes

	Dimensions
	Unit: mm 

	[image: image4.jpg]





	*Unit: mm; Tolerance: ±0.3mm Except Specified
*Housing Material: Black LCP

*Terminal plate: 3 soldering pads, tin Plating Brass


	Electrical And Acoustical Measuring Condition

	Recommended Driving Circuit
[image: image2.png]1.32.Standard text Fixture

1.31.Recommended Circuit
the folowing are examples of extemaly diven crcuis. No reaction in space with in 400mmin all drecton

red
buzzer

Square o

signal
generator | |

400mm








	Frequency Response
      [image: image3.png]200Hz 300 408 S00 600 6808 1k 2k 3k 4k Sk 6k 7k 8k 18k







APPROVAL SHEET


FOR


BUZZER SPECIFICATION











深圳电子批发商城                                                                       https://shop289834711.taobao.com

