
	[image: image4.png]100Hz 200 300 400 500 600 2 3k 4K 5K 6K 7K 8K 10K




[image: image1.jpg]



广州粤能科技有限公司

GUANGZHOU YUENENG TECHNOLOGY CO., LTD.


	CUSTOMER:
	

	MODEL NO.:
	YN-YDW2207-4012S

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED
CHECKED
Add: Room 541, Qiaoxing Business Building, Nanda Road Intersection, No. 33 Xingnan Avenue, Nancun Town, Panyu District, Guangzhou City, Guangdong Province,China.

Mobile:+86-18924165988            skype:18924165988

WhatsApp: +86-18924165988         Email: cngzyn@163.com 




	Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060mbar) unless the standard condition (Temperature: 25±2℃, Humidity: 60%～70 %R.H. Atmospheric pressure: 860 ～1060mbar) is regulated to measure.

	1
	Rated Voltage
	12Vo-p

	2
	Max Input Voltage
	30Vo-p

	3
	Rated Current
	Max.12mA

	4
	Sound Output at 10cm
	Min.95dB

	5
	Resonant Frequency
	4kHz±0.5

	6
	Capacitance at 100Hz (pF)
	19000±30%

	7
	Operating Temperature
	-30℃～+85℃

	8
	Store Temperature
	-40℃～+90℃

	9
	Net Weight
	Approx 2.3g

	10
	size
	22*7mm

	11
	Housing Material 
	PPS

	12
	RoHS
	YES

	Dimensions
	Unit: mm

	[image: image2.png]®3.9 Sound hole 3.3

Recommended PCB
5 Foot Pattern
m
] I
5 = 7 i
IS
14
Y M DD 7:0.3] 243




 

	*Unit: mm; Tolerance: ±0.3mm Except Specified

*Housing Material: Black PPS
*Terminal plate: 2 soldering pads, tin Plating Brass


	3.RECOMMEND DIRVING CIRCUIT
 [image: image3.png]Bx

L oo, 12000 EREBEER
q & Uz |pooo| | RBER
0888 &8
© | 85dB r

®
RN SE——
EE | L
| i

| BISAEERE 10CM




a


	4.Frequency Response



APPROVAL SHEET


FOR


SMD BUZZER











Guangzhou Yueneng Technology Co., Ltd.  
Guangzhou Yueneng Technology Co., Ltd.  

