
	[image: image1.jpg]



[image: image2.png]L=150=10mm| fi

78 [TEE




广州粤能科技有限公司

GUANGZHOU YUENENG TECHNOLOGY CO., LTD.

	CUSTOMER:
	

	MODEL NO.:
	YN-YDY5530-3512W150

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED
CHECKED
Add:Room 413, Qiaoxing Business Building, Nanda Road Intersection, No. 33 Xingnan Avenue, Nancun Town, Panyu District, Guangzhou City, Guangdong Province，China.
Mobile:+86-18924165988            WhatsApp: +86-18924165988         

skype:18924165988                 Email: cngzyn@163.com  


	Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060hPa) unless the standard condition (Temperature: 25±3℃, Humidity: 60±10%R.H. Atmospheric pressure: 860 ～1060hPa) is regulated to measure.

	1
	Rated Voltage
	12Vo-p  

	2
	Operating Voltage
	3-16Vo-p

	3
	Rated Current
	Max.60mA 

	4
	Sound Output at 10cm
	Min. 100dB;

	5
	Resonant Frequency
	3.5kHz±0.5

	6
	Operating Temperature
	-20℃～+70℃

	7
	Store Temperature
	-30℃～+80℃

	8
	Net Weight
	Appro x 

	9
	Tone Nature
	Alarm sound

	10
	Dimension
	Φ54.6 x31Hmm

	Dimensions
	Unit: mm 

	

	


YNKJ





BUZZER SPECIFICATION












