

	[image: image1.jpg]






	CUSTOMER:
	

	MODEL NO.:
	YN-YDY-3015A-3512W-110

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED
CHECKED



	Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060hPa) unless the standard condition (Temperature: 25±3℃, Humidity: 60±10%R.H. Atmospheric pressure: 860 ～1060hPa) is regulated to measure.

	1
	Rated Voltage
	12V

	2
	Operating Voltage
	3~24V

	3
	Rated Current(mA)
	≤10

	4
	Sound Output at 10cm
	≥90

	5
	Housing Material
	ABS(White)

	6
	Resonant Frequency
	3500±500Hz（IN）

	7
	Operating Temperature
	-20℃～+70℃

	8
	Store Temperature
	-30℃～+80℃

	9
	Net Weight
	Approx 7.1g

	10
	Tone Nature
	Constant

	11
	Dimension
	Φ30x H15 mm Lead wire

	12
	Rohs
	Yes

	Dimensions
	Unit: mm 

	[image: image2.png]; ;.K.K»_.







	*Unit: mm; Tolerance: ±0.3mm Except Specified

*Housing Material: Black LCP

*Terminal plate: 3 soldering pads, tin Plating Brass


	Electrical And Acoustical Measuring Condition

	Recommended Driving Circuit
[image: image3.png]1.32.Standard text Fixture

1.31.Recommended Circuit
the folowing are examples of extemaly diven crcuis. No reaction in space with in 400mmin all drecton

red
buzzer

Square o

signal
generator | |

400mm








	Frequency Response
      
[image: image4.emf]0

10

20

30

40

50

60

70

80

90

100

110

120

3V 5V 7V 9V 12V 24V

dB-V mA-V





YN





BUZZER SPECIFICATION











深圳电子批发商城                                                                       https://shop289834711.taobao.com

_1234567890.xls
Sheet1

		

						3V		5V		7V		9V		12V		24V

				dB-V		100		109		112		113		116		117

				mA-V		2		3		4.7		5.4		9		18.6





Sheet1

		



dB-V

mA-V




