

	[image: image1.jpg]




GUANGZHOU YUENENG TECHNOLOGY CO., LTD.

	CUSTOMER:
	

	MODEL NO.:
	YN-YDY2316-3505P-17

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED
CHECKED




	Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060hPa) unless the standard condition (Temperature: 25±3℃, Humidity: 60±10%R.H. Atmospheric pressure: 860 ～1060hPa) is regulated to measure.

	1
	Rated Voltage
	12Vo-p;

	2
	Operating Voltage
	3-24Vo-p;

	3
	Rated Current
	 Max:15mA;

	4
	Sound Output at 10cm
	Min. 85dB;

	5
	Capacitance at 120Hz
	\

	6
	Resonant Frequency
	3500±500Hz（IN）

	7
	Operating Temperature
	-35℃～+75℃

	8
	Store Temperature
	-40℃～+85℃

	9
	Net Weight
	Approx 5.2g

	10
	Tone Nature
	Constant

	11
	Dimension
	Φ23.2x H16 mm 

	12
	R o H S
	Yes

	Dimensions
	Unit: mm 

	[image: image2.jpg]





	*Unit: mm; Tolerance: ±0.3mm Except Specified
*Housing Material: Black LCP

*Terminal plate: 3 soldering pads, tin Plating Brass


	Electrical And Acoustical Measuring Condition

	Recommended Driving Circuit
[image: image3.png]1.32.Standard text Fixture

1.31.Recommended Circuit
the folowing are examples of extemaly diven crcuis. No reaction in space with in 400mmin all drecton

red
buzzer

Square o

signal
generator | |

400mm








	Frequency Response
[image: image4.png]Voltage vs. Sound Pressure Level

ure Level (dB)
88 5 &

8

Sound Pressyre Level

at

10 cm, 25°C

Tewp 257
at 10en

2

5

8

11 14 17 20 23
Voltage (vdc)

Current Consumption (mA)

Voltage vs. Current Consumption

25

20
15

10

at 25°C

Teap 25"
at 1om

2

58 11 14 17 20 23
Voltage (Vdc)






Packaging

[image: image5.png]»
4
Q
<]
3

5x10PCS:

Tray Size: 315 x 170 x 28 mm
Tray QTY: 50 pes per tray

Carton Size: 520 x 330 x 280 mm
Carton QTY: 1,500 pes per carton




[image: image6.png](3PCS Plastic Bag)

Carton Box

=
o
¢
2
ot
S3
Du_.r
28
%P
Pm





YNKJ





BUZZER SPECIFICATION








深圳电子批发商城                                                                       https://shop289834711.taobao.com
GUANGZHOU YUENENG TECHNOLOGY CO., LTD.                                                 

