

	[image: image1.jpg]




GUANGZHOU YUENENG TECHNOLOGY CO., LTD.

	CUSTOMER:
	

	MODEL NO.:
	YN-YDY2319-3712P-15

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED

CHECKED





	Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060hPa) unless the standard condition (Temperature: 25±3℃, Humidity: 60±10%R.H. Atmospheric pressure: 860 ～1060hPa) is regulated to measure.

	1
	Rated Voltage
	12Vo-p;

	2
	Operating Voltage
	3-24Vo-p;

	3
	Rated Current
	 Max:12mA;

	4
	Sound Output at 10cm
	Min 90dB Typical 95dB

	5
	Capacitance at 120Hz
	\

	6
	Resonant Frequency
	3700±500Hz（IN）

	7
	Operating Temperature
	-10℃～+60℃

	8
	Store Temperature
	-20℃～+70℃

	9
	Net Weight
	Approx 5g

	10
	Tone Nature
	Constant

	11
	Dimension
	Φ23x H19 mm

	12
	R o H S
	Yes

	Dimensions
	Unit: mm 

	[image: image2.png]23

St
80¢

19

8'€ce






	*Unit: mm; Tolerance: ±0.3mm Except Specified
*Housing Material: Black LCP

*Terminal plate: 3 soldering pads, tin Plating Brass


	Electrical And Acoustical Measuring Condition

	Recommended Driving Circuit
[image: image3.png]1.32.Standard text Fixture

1.31.Recommended Circuit
the folowing are examples of extemaly diven crcuis. No reaction in space with in 400mmin all drecton

red
buzzer

Square o

signal
generator | |

400mm








	Frequency Response
[image: image4.png]dB 100






YNKJ





BUZZER SPECIFICATION








深圳电子批发商城                                                                       https://shop289834711.taobao.com
GUANGZHOU YUENENG TECHNOLOGY CO., LTD.                                                 

