
	[image: image7.png]



[image: image1.jpg]



广州粤能科技有限公司

GUANGZHOU YUENENG TECHNOLOGY CO., LTD.

	CUSTOMER:
	

	MODEL NO.:
	YN-YDY3019-3512P-15

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED
CHECKED
Add: ,132 Shishu Road, Yuexiu District, Guangzhou City, Guangdong China.

Mobile:+86-18924165988            WhatsApp: +86-18924165988         

skype:18924165988                 Email: cngzyn@163.com  


	Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060hPa) unless the standard condition (Temperature: 25±3℃, Humidity: 60±10%R.H. Atmospheric pressure: 860 ～1060hPa) is regulated to measure.

	1
	Rated Voltage (VDC)
	12

	2
	Operating Voltage(VDC)
	3～20

	3
	*Rated Current(mA)
	≤12

	4
	*Sound Output at 30 cm(dB)
	≥100

	5
	*Resonance Frequency (Hz)
	3500±500

	6
	Tone
	  Continuous 

	7
	Operating Temperature(℃)
	-40～+85

	8
	Storage Temperature(℃)
	-40～+90

	9
	Weight (g)
	9.3

	10
	Housing Material
	PBT

	Dimensions
	Unit: mm 

	[image: image2.png]20.2

2-@0.8

230.2+0.3
15+0.3

19.2:03 |5

6.9 Sound hole





[image: image6.png]




	*Unit: mm; Tolerance: ±0.3mm Except Specified
*Housing Material: Black LCP

*Terminal plate: 3 soldering pads, tin Plating Brass


	Electrical And Acoustical Measuring Condition

	Recommended Driving Circuit
[image: image3.png]5Yp-p

Signal in
>

o

[N4148 A Buzzer

W

470

L 1503199





Resonant frequency, 1/2 duty cycle. Square wave.

Signal amplitude should be large enough to saturate the transistor.

	Recommended Setting


[image: image4.wmf]4

0

c

m

1

0

c

m

M

i

c

.

S

P

L

d

B

测

试

示

意

图

A

m

p

l

i

f

i

e

r

放

大

器

S

M

-

1

6

4

1

F

U

N

C

T

I

O

N

G

E

N

E

R

A

T

O

R

信

号

发

生

仪

P

o

w

e

r

电

源

B

u

z

z

e

r




	SPL & Current Consumption vs. Input Voltage:
[image: image5.png]dB/30cm

110
105

100

95
90

85
80

SPL|

//

L

/

Il

mA
24
20
16
12

8
4

0

3 6 9 1215 18 21VDC




Packing:

NOTES:

1. 50 PCS per box

2.Total 20 boxes per carton

3.Total 1000 PCS carton
4.Volume:40*36*44.5cm




YNKJ





BUZZER SPECIFICATION












_1234567890.bin

