[image: image1.jpg]98203

70203

50:03
o —

29072002






	[image: image3.jpg]



[image: image4.png]



GUANGZHOU YUENENG TECHNOLOGY CO., LTD.

	CUSTOMER:
	

	MODEL NO.:
	YN1007A40Khz  T 

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED

CHECKED

Add: Room 413, Qiaoxing Business Building, Nanda Road Intersection, No. 33 Xingnan Avenue, Nancun Town, Panyu District, Guangzhou City, Guangdong Province

Mobile:+86-18924165988         skype:18924165988

WhatsApp: +86-18924165988     Email: cngzyn@163.com



	Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060hPa) unless the standard condition (Temperature: 25±3℃, Humidity: 60±10%R.H. Atmospheric pressure: 860 ～1060hPa) is regulated to measure.

	1
	Function
	Transmit 

	2
	structure
	waterproof 

	3
	Housing Material
	Aluminum 

	4
	Center Frequency
	40±1.5KHz

	5
	Free Capacitance
	2200±20%Pf 

	6
	Sound Pressure Level
	115dB min.(0dB=0.02mPa)

	7
	-6 dB Directivity deg
	70±15°

	8
	Ringing
	2.0ms max

	9
	Sensitivity
	-67dB Min.(0dB=1v/μ bar)

	10
	Operating Temperature
	-30℃～+70℃

	11
	Storage Temperature
	-40℃～+80℃

	12
	Maximum allowable voltage
	120Vp-p

	13
	Weight
	1g

	Dimensions
	Unit: mm 

	[image: image5.png]=/






	*Unit: mm; Tolerance: ±0.3mm Except Specified
*Housing Material: Aluminum 
*Terminal plate: 3 soldering pads, tin Plating Brass


	Electrical And Acoustical Measuring Condition

	Recommended Driving Circuit
[image: image2.png]Receiver

e

W me
=
[

7c|  AnechanRoom

Transmiter

Tus.  mc
=}

1=t

4®

F-30cm-+f

FC

‘AnechonRoom  04B-0.02nPa

Input Vokage: 10Vims.

A Osclltor B Ampifier C: Vokmeter R 38K MC: Mirophane TW: Tweeter

RUS: Recelver Utrasanic Sensor TLLS: Transiter Uirasonic Sensor  F.C. Freguency Courter







	Package Specification

1.Each foam box has 50 products

2.50 foam boxes in each case

3.2500 products per carton

4.Packing case size 520X420X280 mm





Specifications


For


Ultrasonic transducer











深圳电子批发商城                                                                       https://shop289834711.taobao.com
GUANGZHOU YUENENG TECHNOLOGY CO., LTD.                                                 

