
	[image: image9.emf]


[image: image1.jpg]



GUANGZHOU YUENENG TECHNOLOGY CO., LTD.


	CUSTOMER:
	

	MODEL NO.:
	                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                 

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED

CHECKED

Add: Room 413, Qiaoxing Business Building, Nanda Road Intersection, No. 33 Xingnan Avenue, Nancun Town, Panyu District, Guangzhou City, Guangdong Province

Mobile:+86-18924165988            WhatsApp: +86-18924165988         

skype:18924165988                 Email: cngzyn@163.com 




	 SPECIFICATION
	MODEL NO.
	YN1206-3220 弹片听筒
	Total Page

5

	ISSUED DATE
	2011/07/17
	LATEST. NO.
	2025/07/18
	

	1. SCOPE
THIS SPECIFICATION IS APPLIED TO THE DYNAMIC RECEIVER/SPEAKER WHICH IS USED IN CELLULAR PORTABLE PRODUCT.

2. GENERAL SPECIFICATIONS

1. TYPE
earpiece
2. OUT SIDE SIZE
12*6*2.1Hmm , Shown  Fig.3
3. IMPEDANCE
32Ω±15%  at 1 KHz  at 1 KHz 179mVms Sine Wave
4. RESONANCE FREQUENCY
850±20% Hz @1khz 10mw 179mV Sine wave
5.SENSITIVITY (S.P.L.)
110dB ±3dB at 1kHz, 10mw 179mVms, IEC318ear
6. RATED / MAX. POWER
20mw /30mw

7. FREQUENCY RANGE
fo ~ 20kHz
8. DISTORTION
Less than10% at 1000Hz 179mV
9. UNIT WEIGHT
0.5g±0.1g
10. MEASURING DIAGRAM
Shown in Fig.1
11. FREQUENCY RESPONSE
Shown in Fig.2

12. OPERATING TEMPERATURE
- 30℃ ～ +65℃

13. STORAGE TEMPERATURE
- 40℃ ～ +65℃

14. P0LARITY
No one should perceive noise of sine wave 2.0V of 0.3kHz ~ 5kHz .

15.OPERATION TEST

The measured speaker is fixed, voltage0.85V sinusoidal sweep frequency signal, frequency range of FO-20 kHz, scan 1S, 5-10cm from the human ear, the monitoring time shall not be less than 3 cycles.



	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	3. ENVIRONMENTAL CHARACTERISTICS
RESULTS OF AFTER TEST
Sensitivity difference shall be within (3dB) and should satisfy the test  described  under  item 14  after each following test.

1. APPEARANCE 
Should not exist any obstacle to be harmful to normal operation : damages \ cracks \ rusts and distortions etc.

3. POLARITY
When positive voltage is applied to the terminal marked (+), diaphragm should move to the front.

4. LOAD TEST
0.5 W white noise is applied for 96 hours and satisfy the test listed on item 05,08,14.

5. HIGH TEMPERATURE DURABILITY
After having been in a test chamber for 96 hours at the  condition of +65℃ ±5℃ and then let 2 hours in a room .

6. COLD TEMPERATURE DURABILITY
After having been in a test chamber for 96 hours at  the condition  of -40℃ ±5℃ and then let 2 hours in a room .

7. HUMIDITY DURABILITY
After having been in a test chamber for 96 hours at the condition  of  +40℃±5℃ 90%~96% RH  and then  let 2 hours in a room .

8.TEMPERATURE  CYCLE DURABILITY
Low temperature  -40 ℃ ±2℃ (2 hour ) /  high temperature        +65±2℃ (2 hour ) : 1 cycle , total 10 cycles and then let 2 hours in a room .

9.VIBRATION DURABILITY
Frequency Range:10Hz ~ 500Hz 

Amplitude : 0.35mm or acceleration : 50m/s2

Duration : 2h per axis=10 cycles ; 3 axis

10.DROP TEST
The speaker unit shall stand 10 random drops from a height of 1 meter to a concrete floor faced with 5m/m thick hard wood board and be nothing mechanical damage.

11.PACKING STRENGTH
As per packing condition, from height of 1 meter, the unit is to be dropped 1 time each in three planes on concrete floor.



	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


	4.STANDARD TEST CONDITION OF SPEAKER (Fig.1)[image: image4]
(SPEAKER MODE)  (Fig.2)

[image: image2.png]SPL VOL:179mV DIS:10MM

145

135

125

115

105

EE

100

1000

10000






	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	4.ASSEMBLY DIMENSIONS (Fig.3)
[image: image3.png]O.5XU i el

60,2

260.2

4 =

4

T0.50.7

21






	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	4.PACKING (Fig.4)

[image: image5]


	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


Specifications


for


Speaker








� EMBED  \* MERGEFORMAT ���





MULTI FUNCTION TRANSDUCER�TYPE     :   1206AA   �QUANTITY :   5850PCS�NET   WT :   3.0KG�GROSS WT :   6.5KG�VOLUME   :   45*29*38CM





� EMBED  \* MERGEFORMAT ���





� EMBED  \* MERGEFORMAT ���





225PCS /1板�5850PCS/1箱 








[image: image6][image: image7.wmf]INPUT=179mV

TEST MODEL

EAR PIECE

MICROPHONE

B&K  4192

IEC  318  COUPLER

B&K  4153

AMPLIFIER

ANALYZER

INPUT

OUTPUT

PRINTER

B&K 2012

[image: image8.emf]

_1234567891.dwg

_1234567892.dwg

_1234567890.dwg

