
	[image: image5.png]14.240.

|





[image: image1.jpg]



GUANGZHOU YUENENG TECHNOLOGY CO., LTD.


	CUSTOMER:
	

	MODEL NO.:
	YN20D-0801W80

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED

CHECKED

Add: Room 413, Qiaoxing Business Building, Nanda Road Intersection, No. 33 Xingnan Avenue, Nancun Town, Panyu District, Guangzhou City, Guangdong Province

Mobile:+86-18924165988            WhatsApp: +86-18924165988         

skype:18924165988                 Email: cngzyn@163.com 




	 SPECIFICATION
	MODEL NO.
	YN20D-0801W80
	Total Page

4

	ISSUED DATE
	2011/07/18
	SPEC. NO.
	2025/05/20
	

	1. SCOPE
THIS SPECIFICATION IS APPLIED TO THE DYNAMIC RECEIVER/SPEAKER WHICH IS USED IN CELLULAR PORTABLE PRODUCT.

2. GENERAL SPECIFICATIONS

ANALYSIS  ITEMS

SPECIFICATIONS

1. TYPE
Dynamic speaker
2. OUT SIDE SIZE
Ф20mm , Shown  Fig.3
3. IMPEDANCE
8Ω±15%  at 1 KHz 1.0Vms Sine Wave
4. RESONANCE FREQUENCY
650±20% Hz
at 1.0V Sine wave
5.SENSITIVITY (S.P.L.)
90±3dB SPL At800,1000,1200,1500Hz in average (0dB SPL=20μPa) Input voltage: 0.1W(Sine wave)0.1M Measured baffler recommended.
6. RATED / MAX. POWER
0.5W / 0.8W      
7. FREQUENCY RANGE
fo ~ 20kHz，
8. DISTORTION
Less than 8 % at 1000Hz 1 W
9. UNIT WEIGHT
2.0g±0.2g
10. MEASURING DIAGRAM

Shown in Fig.1

11. FREQUENCY RESPONSE
Shown in Fig.2
12. OPERATING TEMPERATURE
- 30℃ ～ +70℃
13. STORAGE TEMPERATURE
- 40℃ ～ +80℃
14. OPERATION TEST
Buzz,Rattle,etc Should not be audible at 2.5V sine wave from 200 Hz to 5K Hz.


	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


	3.STANDARD TEST CONDITION OF SPEAKER (Fig.1)
[image: image2]
4.FREQUENCY RESPONSE (SPEAKER MODE)  (Fig.2)
[image: image3.wmf]Standard Baffler Recommended

In IEC 268-5 Where 1350mm x 1650mm

Power Amplifier

Audio Analyzer

Microphone

B&K Type 4192 & 2669

Speaker

Model 

B&K Type 2012


[image: image4.png]48888Hz

H98.38 dB
H91.82 dB

Fe

EEE
HEE
16Hz

1

0 1KH=z
I
| O §

dm3 =0

Y.
=H1Hd cm

DCR
Ueq
i<t .

=1

YTZZEHE8H4



5.ASSEMBLY DIMENSIONS (Fig.3)



	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


Specifications


for


Speaker








� EMBED  \* MERGEFORMAT ���








GUANGZHOU YUENENG TECHNOLOGY CO., LTD.

_1234567890.dwg

