
	[image: image1.wmf]Standard test condition of speaker

Speaker

Power Amplifier

Audio Analyzer

CRY6151

Microphone

318


[image: image4.jpg]



广州粤能科技有限公司
GUANGZHOU YUENENG TECHNOLOGY CO., LTD.


	CUSTOMER:
	

	MODEL NO.:
	YN3020-0801W40

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED

CHECKED

Add: ,132 Shishu Road, Yuexiu District, Guangzhou City, Guangdong China.

Mobile:+86-18924165988            WhatsApp: +86-18924165988         

skype:18924165988                 Email: cngzyn@163.com  



	SPECIFICATION
	MODEL NO.
	YN3020-0801W40
	Total Page

5

	ISSUED DATE
	2011/12/02
	SPEC. NO.
	2023/05/25
	

	1. SCOPE

This specification covers our product of dynamic speaker unit for mobile telephone use.

2. GENERAL SPECIFICATIONS

ANALYSIS  ITEMS

SPECIFICATIONS

1. TYPE
Dynamic speaker
2. OUT SIDE SIZE
20mm×30mm , Shown  Fig.3
3. IMPEDANCE
8Ω±15%  at 1 KHz 1Vms Sine Wave
4. RESONANCE FREQUENCY
700±20% Hz
at 1.0V Sine wave
5.SENSITIVITY (S.P.L.)
90dB ±3dB at 0.5w/ 0.1m / 1.0,1.2,1.5,2.0×1000Hz average
6. RATED / MAX. POWER
1.0W / 1.2W
7. FREQUENCY RANGE
fo ~ 20kHz
8. DISTORTION
Less than 15 % at 1000Hz 0.6 W
9. UNIT WEIGHT
2.5g±0.1g
10. MEASURING DIAGRAM

Shown in Fig.1
11. FREQUENCY RESPONSE
Shown in Fig.2
12. OPERATING TEMPERATURE
- 30℃ ～ +70℃
13. STORAGE TEMPERATURE
- 40℃ ～ +85℃
14. OPERATION TEST
No one should perceive noise of sine wave 2.83V of 0.3kHz ~ 5kHz .


	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


	3. ENVIRONMENTAL CHARACTERISTICS

RESULTS OF AFTER TEST
Sensitivity difference shall be within (3dB) and should satisfy the test  described  under  item 14  after each following test.
1. APPEARANCE 
Should not exist any obstacle to be harmful to normal operation : damages \ cracks \ rusts and distortions etc.
3. POLARITY
When positive voltage is applied to the terminal marked (+), diaphragm should move to the front.
4. LOAD TEST
0.8 W white noise is applied for 96 hours and satisfy the test listed on item 05,08,14.
5. HIGH TEMPERATURE DURABILITY
After having been in a test chamber for 96 hours at the  condition of +60℃ ±3℃ and then let 2 hours in a room .
6. COLD TEMPERATURE DURABILITY
After having been in a test chamber for 96 hours at  the condition  of -20℃ ±3℃ and then let 2 hours in a room .
7. HUMIDITY DURABILITY
After having been in a test chamber for 96 hours at the condition  of  +40℃±2℃ 90%~96% RH  and then  let 2 hours in a room .
8.TEMPERATURE  CYCLE DURABILITY

Low temperature  -20 ℃ ±2℃ (2 hour ) /  high temperature        +60±2℃ (2 hour ) : 1 cycle , total 10 cycles and then let 2 hours in a room .

9.VIBRATION DURABILITY

Frequency Range:10Hz ~ 55Hz ~10Hz

Amplitude : 1.5mm or acceleration : 50m/s2

Duration : 2h per axis=10 cycles ; 3 axis

10.DROP TEST

The speaker unit shall stand 10 random drops from a height of 1 meter to a concrete floor faced with 5m/m thick hard wood board and be nothing mechanical damage.

11.PACKING STRENGTH

As per packing condition, from height of 1 meter, the unit is to be dropped 1 time each in three planes on concrete floor.



	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


	4.STANDARD TEST CONDITION OF SPEAKER (Fig.1)

4.1. STANDARD TEST CONDITIONS：

      Temperature:         15℃～35℃

      Relative Humidity:    45%～85%(RH)

      Air Pressure:         860mhpa～1060mhpa

    4.2. JUDGEMENT TEST CONDITIONS：

      Temperature:         20±2℃

      Relative Humidity:    60%～70%(RH)

      Air Pressure:         860mhpa～1060mhpa

4.3. SPEAKER MODE MEASURING CIRCUIT


[image: image5.png]Loudspeaker Responses
120

Fandwenta 01 [0

115

110

105

100-]

a5-|

SEL [ re 20u Pa]

e5-7 ]

60-|

Frequency

SPL (4 re

1000
Frequancy [H:]

Fundanentel

Ik

45,4262

Fundunentel

E]

75,6064

Delts

a0

-1a.5188





EAKER MODE)  (Fig.2)




	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	4.ASSEMBLY DIMENSIONS (Fig.3)

Unless otherwise specified,tolerance:±0.5 (unit:mm)

[image: image2.png]4.0+0.3

4.3+0.3

C0F6'vL







	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	4.PACKING (Fig.4)


[image: image3.wmf]¡Á

10=1

5

00PCS

¡Á

4

=

6

000PCS

325

245



	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


APPROVAL SHEET


FOR


speaker











GUANGZHOU YUENENG TECHNOLOGY CO., LTD.


_1234567890.dwg
Administrator


_1234567891.dwg
Administrator


