[image: image1.jpg]




	[image: image7.png][o1/18,2010 11:21 cRvelzsY 7 5 B W i 0 var | [®aE

f 0 100BHzE&E ¢ 076.28 dB Acz: 2
a3 >0 FHTEE : 082.89 B Zn 2
=108 cn Fi= 10tz F2=48000Hz 8 : Hz

0 a8 FE{Eh ¢ 20.0 pPa  IEE :

]

070

200Hz 300 460 500 800 1KkHz 2k 3k 4k Sk 6k 7k 9k18k 15k 20Kk





GUANGZHOU YUENENG TECHNOLOGY CO., LTD.


	CUSTOMER:
	

	MODEL NO.:
	YN36D5.4H  32Ω 20mW

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED

CHECKED

Add: ,132 Shishu Road, Yuexiu District, Guangzhou City, Guangdong China.

Mobile:+86-18924165988            WhatsApp: +86-18924165988         

skype:18924165988                 Email: cngzyn@163.com 

	


	SPECIFICATION
	MODEL NO.
	YN36D5.4H  32Ω 20mW
	Total Page

4

	ISSUED DATE
	2022/09/29
	LATEST. NO.
	2022/11/16
	

	1. SCOPE

This specification is a typical speaker unit for telephone hand set.

2. GENERAL SPECIFICATIONS

ANALYSIS  ITEMS

SPECIFICATIONS

1. TYPE
dynamic earphone speaker
2. OUT SIDE SIZE
Φ36 H5.4 , Shown  Fig.3
3. IMPEDANCE
32Ω±15% at 1V 1khz
4. RESONANCE FREQUENCY
350HZ±20% Hz at 1V 1khz
5.SENSITIVITY (S.P.L.)
120±3 at 1mW 0.01Meter.  At 1000Hz.
6. RATED / MAX. POWER
20mW / 30mW
7. FREQUENCY RANGE
F0( 2.0KHz
8. DISTORTION
Less than 5 % Rated power input at 1KHz. 
9. UNIT WEIGHT
APPROX.8.4 grams
10. MEASURING DIAGRAM

Shown in Fig.1
11. FREQUENCY RESPONSE
Shown in Fig.2
12. OPERATING TEMPERATURE
- 20℃ ～ +55℃
13. STORAGE TEMPERATURE
- 25℃ ～ +65℃
14. OPERATION TEST
Must be free audible noise (buzzes and rattles)

(0( 20KHz frequency range ,input level up to 2Vrms)


	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


	Reliable Test

After any following test leave speakers at room temperature for 1 hour, SPL shall not deviate by±3 db from initial value

3-1
Load Test
white  noise for 96 hours  at  20mW input power.
3-2
High Temperature Test
+65±2 °C  96hours 
3-3
Low Temperature  Test
 -25±2 °C  96hours 
3-4
Humidity Test
+40±2 °C  relative humidity 90~95%R.H  96hours
3-5
Vibration Test
Frequency 30±15 Hz, Amplitude 1.5 mm for 2 Hours per axis(x. y. z axis)
3-6
Temperature Cycle test
( 25( + 65 (C   5 Cycles Temperature test.
Speaker Construction

NO
PART NAME
MATERIAL
Q`TY
1
TERMINAL
FR-4
1
2
YOKE
SPCC
1
3
MAGNET
Nd-Fe-B
1
4
WASHER
SPCC
1
5
CLOTH
CLOTH
1
6
VOICE COIL
COPPER
1
7
DIAPHRAGM
PEN
1
8
FRAME
SPCC
1

[image: image2.wmf]

	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


	4.STANDARD TEST CONDITION OF SPEAKER (Fig.1)
[image: image3.png]Anechoic Chamber




EAKER MODE)  (Fig.2)

 SHAPE  \* MERGEFORMAT 




[image: image5.png]DCR:
2080Hz- 2008Hz)

108BHz Sens.= 1Z8.
fiverage .SPL = 117.

Headphone Frequency Response Test

36.60 Pass
Sueep Speed: 1712 Oct.

Att. @ +8dB N 1.18

S6Hz 18BHz 288Hz

Voltage applied for |EE-179.0mV

2KHz

28Hz

SKHz 18KHz

1688NH

[Stop F. EUA

20K 308K





	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	4.ASSEMBLY DIMENSIONS (Fig.3)
[image: image6.png]16: Drawing(Fig1)  Unit: mm  Tolerance:+0.3mm

Rﬂﬂ:ﬂ *N.oﬁxﬁsl
il i+
e

)
I 1
l——2'0F9'FCo \Li
-2 0F0 900 —=






	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


SPEAKER


SPECIFICATION








深圳电子批发商城                                                                       https://shop289834711.taobao.com

_1234567890.dwg

