
	[image: image5.wmf]Standard Baffler Recommended

In IEC 268-5 Where 1350mm x 1650mm

Power Amplifier

Audio Analyzer

Microphone

B&K Type 4192 & 2669

Speaker

Model 

B&K Type 2012


[image: image1.jpg]



GUANGZHOU YUENENG TECHNOLOGY CO., LTD.


	CUSTOMER:
	

	MODEL NO.:
	YN4140-2005

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED

CHECKED

Add: ,132 Shishu Road, Yuexiu District, Guangzhou City, Guangdong China.

Mobile:+86-18924165988            WhatsApp: +86-18924165988         

skype:18924165988                 Email: cngzyn@163.com 




	 SPECIFICATION
	MODEL NO.
	YN4140
	Total Page

6

	ISSUED DATE
	2011/07/18
	LATEST NO.
	YN4140-2005
	

	1. SCOPE
THIS SPECIFICATION IS APPLIED TO THE DYNAMIC RECEIVER/SPEAKER WHICH IS USED IN CELLULAR PORTABLE PRODUCT.

2. GENERAL SPECIFICATIONS

ANALYSIS  ITEMS

SPECIFICATIONS

1. TYPE

ultrasonic horn

2. OUT SIDE SIZE

Ф41*H14.5 mm , Shown  Fig.3

3. Operating Voltage

1-30
4. Rated voltage

5V

5.Max. Current Consumption at Rated Voltage (mA)

≤10mA
6.electrostatic capacity
100±30%nF

7.Resonant Frequency Range
2.5-60  20KHZ
8.Operating Frequency
25±5

9.Sound pressure level
MIN.90
10.Operating temp.
-20~~+70

11.Storage temp.
-30~~+70

12.Weight (MAX)
/g

13.Housing Materail

ABS



	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	3. ENVIRONMENTAL CHARACTERISTICS

RESULTS OF AFTER TEST
Sensitivity difference shall be within (3dB) and should satisfy the test  described  under  item 14  after each following test.
1. APPEARANCE 
Should not exist any obstacle to be harmful to normal operation : damages \ cracks \ rusts and distortions etc.
3. POLARITY
When positive voltage is applied to the terminal marked (+), diaphragm should move to the front.
4. LOAD TEST
0.5 W white noise is applied for 96 hours and satisfy the test listed on item 05,08,14.
5. HIGH TEMPERATURE DURABILITY
After having been in a test chamber for 96 hours at the  condition of +80℃ ±5℃ and then let 2 hours in a room .
6. COLD TEMPERATURE DURABILITY
After having been in a test chamber for 96 hours at  the condition  of -40℃ ±5℃ and then let 2 hours in a room .
7. HUMIDITY DURABILITY
After having been in a test chamber for 96 hours at the condition  of  +40℃±5℃ 90%~96% RH  and then  let 2 hours in a room .
8.TEMPERATURE  CYCLE DURABILITY
Low temperature  -40 ℃ ±2℃ (2 hour ) /  high temperature        +85±2℃ (2 hour ) : 1 cycle , total 10 cycles and then let 2 hours in a room .
9.VIBRATION DURABILITY
Frequency Range:10Hz ~ 500Hz 

Amplitude : 0.35mm or acceleration : 50m/s2

Duration : 2h per axis=10 cycles ; 3 axis
10.DROP TEST
The speaker unit shall stand 10 random drops from a height of 1 meter to a concrete floor faced with 5m/m thick hard wood board and be nothing mechanical damage.
11.PACKING STRENGTH
As per packing condition, from height of 1 meter, the unit is to be dropped 1 time each in three planes on concrete floor.


	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


	STANDARD TEST CONDITION OF SPEAKER (Fig.1)

The Speaker shall be mounted in a baffle described in the Fig.1,and tested with 2V(0.1W);
The measuring microphone shall be placed 10cm from the speaker in an anechoic chamber.
[image: image4]
FREQUENCY RESPONSE (SPEAKER MODE)  (Fig.2)

The swept sine-wave frequency response of a loudspeaker should ideally not deviate more

than indicated per Fig.2

At0.1m0.1W

[image: image2.png]C. TYPICAL FREQUENCY RESPONSE CURVE

ey 5 EET i B W 2
J£:2838 mU Fc= 20000Hz 112.44dB Fc= 20086Hz f
g2 Fo= 19881Hz 113.26dB Fo=, Hz f

1op 0B B © 20.0 pPa TR : 001 ns Yo R : 1192 oct

aB

128

it
Fwils
AL
15k 28k 25k 30k 35k 40k 45k 50k 68k 65k





ASSEMBLY DIMENSIONS (Fig.3)

[image: image3.png]




	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


Specifications


for


Speaker








� EMBED  \* MERGEFORMAT ���








GUANGZHOU YUENENG TECHNOLOGY CO., LTD.

_1234567890.dwg

