[image: image1.png]Anechoic Chamber





	[image: image6.jpg]




GUANGZHOU YUENENG TECHNOLOGY CO., LTD.


	CUSTOMER:
	

	MODEL NO.:
	YN4230-0403WD-150

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED

CHECKED

Add: ,132 Shishu Road, Yuexiu District, Guangzhou City, Guangdong China.

Mobile:+86-18924165988            WhatsApp: +86-18924165988         

skype:18924165988                 Email: cngzyn@163.com 

	


	SPECIFICATION
	MODEL NO.
	YN4230-0403WD-150
	Total Page

4

	ISSUED DATE
	2014/03/25
	Updated NO.
	2022/05/07
	

	1. SCOPE

This specification is a typical speaker unit for telephone hand set.

2. GENERAL SPECIFICATIONS

ANALYSIS  ITEMS

SPECIFICATIONS

1. TYPE
Dynamic speaker
2. OUT SIDE SIZE
30mm×42mm , Shown  Fig.3
3. IMPEDANCE
4Ω±15%  at 1 KHz 2.0Vms Sine Wave
4. RESONANCE FREQUENCY
550±20% Hz
at 1.0V Sine wave
5.SENSITIVITY (S.P.L.)
90dB ±3dB at 0.5w/ 0.1m / 10.12.15.20×100Hz average
6. RATED / MAX. POWER
3.0W / 3.5W
7. FREQUENCY RANGE
fo ~ 10kHz
8. DISTORTION
Less than 15 % at 1000Hz 0.6 W
9. UNIT WEIGHT
18.0g±0.1g
10. MEASURING DIAGRAM

Shown in Fig.1
11. FREQUENCY RESPONSE
Shown in Fig.2
12. OPERATING TEMPERATURE
- 30℃ ～ +60℃
13. STORAGE TEMPERATURE
- 40℃ ～ +75℃
14. OPERATION TEST
No one should perceive noise of sine wave 4.0V of 0.3kHz ~ 10kHz .


	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


	3. ENVIRONMENTAL CHARACTERISTICS

RESULTS OF AFTER TEST
After any following tests the response at 1KHz shall not deviate more than (3 dB from the initial value
1. APPEARANCE 
Should not exist any obstacle to be harmful to normal operation : damages \ cracks \ rusts and distortions etc.
3. POLARITY
When positive voltage is applied to the terminal marked (+), diaphragm should move to the front.
4. LOAD TEST
Input power: 1W ,white noise 48 hours。
5. HIGH TEMPERATURE DURABILITY
Temperature: +85℃±3℃

Duration:    48 hours
6. COLD TEMPERATURE DURABILITY
Temperature: -40℃±3℃

Duration:    48 hours
7. HUMIDITY DURABILITY
Temperature: +40℃±2℃     Relative Humidity: 90%(95%

Duration:     48hours      (recovery 6 hours)。
8.TEMPERATURE  CYCLE DURABILITY

Temperature: -40℃±3℃         +85℃±3℃  Cycle: 20 cycles

Duration:     2 hour           2 hour  (recovery 2 hours)

+85(C     2Hrs

+25(C              2Hrs                 2Hrs

-40(C                         2Hrs


          20—40minites    1 cycle

9.VIBRATION DURABILITY

Frequency: 10-55Hz.  Amplitude: 1.5mm

Duration:  2 hours in each 3axes。
10.DROP TEST

Height: 1.0M

Cycle:  10cycles。
11.PACKING STRENGTH

As per packing condition, from height of 1 meter, the unit is to be dropped 1 time each in three planes on concrete floor.



	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


	4.STANDARD TEST CONDITION OF SPEAKER (Fig.1)

[image: image7.png][o1/18,2010 11:21 cRvelzsY 7 5 B W i 0 var | [®aE

f 0 100BHzE&E ¢ 076.28 dB Acz: 2
a3 >0 FHTEE : 082.89 B Zn 2
=108 cn Fi= 10tz F2=48000Hz 8 : Hz

0 a8 FE{Eh ¢ 20.0 pPa  IEE :

]

070

200Hz 300 460 500 800 1KkHz 2k 3k 4k Sk 6k 7k 9k18k 15k 20Kk




EAKER MODE)  (Fig.2)

 SHAPE  \* MERGEFORMAT 




[image: image3.png]140
)

JARi

120+

100+

80

60

404

204

i
100Hz 200 500 1k 2 sk 10k 20k






	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	4.ASSEMBLY DIMENSIONS (Fig.3)
[image: image4.png]1.0RREH

e LR E R TR
o 3002 £.0£0.2
? o]
~4
-+
to | ©
6.5:0.2 Ll
ULIS71 S04 HLloL S0k ULLS71 - 30# ULIS71 30%
5 G £LEE 5 b PRAE

15043

% XH-2p~J

i






	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	4.PACKING (Fig.4)


[image: image5.wmf]260

440

210




	NO.
	DATE
	DUE TO
	REVISION
	SIGNATURE

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


Specifications


for


Speaker











深圳电子批发商城                                                                       https://shop289834711.taobao.com

_1234567890.dwg

