
	[image: image1.jpg]



[image: image2.png]12'1.2-10045

reo [
=
=
%% =
%54

osi0s

O350 T





GUANGZHOU YUENENG TECHNOLOGY CO., LTD.


	CUSTOMER:
	

	MODEL NO.:
	YN49mmPVC-L100

	OUR PART NO..:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED

CHECKED

Add: Room 541, Qiaoxing Business Building, Nanda Road Intersection, No. 33 Xingnan Avenue, Nancun Town, Panyu District, Guangzhou City, Guangdong Province,China.
Mobile:+86-18924165988            WhatsApp: +86-18924165988         

skype:18924165988                 Email: cngzyn@163.com 




	SPECIFICATION
	MODEL NO.
	YN49mmPVC-L100
	Total Page

2

	ISSUED DATE
	2012/04/25
	LATEST NO.
	2025/07/02
	

	1. SCOPE
THIS SPECIFICATION IS APPLIED TO THE DYNAMIC RECEIVER/SPEAKER WHICH IS USED IN CELLULAR PORTABLE PRODUCT.
GENERAL SPECIFICATIONS

Customer:
Part No: 
Product: Piezoelectric elements
DT Part No: 49mmPVC-L100
一、Configuration

Appearance and dimension: see the drawing 

Material

Piezoelectric

Configuration

1:plane√            2:discal

Electrode form

A: fullness √        B: feedback

Metal material

T: brass√ G: stainless steel F: fe 

二，Electrical specification

 Resonant frequency (kHz)

FS=2.8±0.5

 Resonant impedance  (Ω) 

Rr≤500Ω

 Main Capacitance at 1000Hz (PF)
Cm=38000±30%

 Operating temperature (℃)

-20～+60

 Storage temperature (℃)

-30～+70

Piezoelectric elements 

Manual Soldering process
Soldering  Parameter

Temp.( 0C)
Time(Sec.)

Times

Inclusive Lead

★
silver electrode
310±10
1.5 second or less.

1~2

★
metal plate
340±10
2 second or less.

1~2
Lead Free

silver electrode
320±10

2 second or less.

1~2

metal plate
350±10

3 second or less.

1~2
Solder Dot:   Diameter MAX 3.0mm,   High MAX 1.5mm, 

Soldering sequence:   1、 silver electrode      2、 metal plate





Specifications


for


buzzer








GUANGZHOU YUENENG TECHNOLOGY CO., LTD.

